# ÉCLAIRE Flux Network
List and Protocols of Mandatory Measurements

Table 1. Mandatory measurements at ECLAIRE flux network

1. Site Description for all sites

| Parameter Type | Parameter | Units |
| --- | --- | --- |
| Site Description | Slope | ‘ vertical |
|  | Elevation | m AMSL |
|  | Latitude | ‘°N |
|  | Longitude | ‘°E |
|  | Annual temperature | °C |
|  | Annual rainfall | Mm |

1a. Forests

| Parameter Type | Parameter | Units |
| --- | --- | --- |
| Management | Age of forest | - |
|  | Details of land use over the last 25 years. | - |
|  | Management scheme | - |
|  | Rotation length | - |
|  | Site preparation (ploughed etc) | - |
|  | Disturbance | Dates and description of main events (including windthrow, fire, others..) |
|  | Thinning | Dates and description |
|  | Harvesting | Dates and description |

1b. Croplands

| Parameter Type | Parameter | Units |
| --- | --- | --- |
| Site Description | Time since the site has been under crops | Years |
|  | Details of crops in the previous 2-3 years |  |
|  | Details of rotation/long-term cropping practice |  |
|  | Crop details – crop type, cultivar |  |
| Sowing | Sowing date | DD/MM/YY |
| Harvest | Dates of harvest | DD/MM/YY |
|  | Yield of harvest | kg m-2 |
| Fertiliser application (mineral) | Dates of each mineral fertilisation | DD/MM/YY |
|  | Amount of N applied at each fertiliser event | (kg N ha-1) |
|  | Amount of phosphorus applied at each fertiliser event | (kg P ha-1) |
|  | Amount of potassium applied at each fertiliser event | (kg K ha-1) |
|  | Chemical form of mineral N, P and K applied (e.g. straight ammonium nitrate, urea, calcium ammonium nitrate, potassium phosphate…) | - |
| Fertiliser application (Organic) | Dates of each organic fertilisation | DD/MM/YY |
|  | Form of organic fertiliser applied at each fertiliser event (e.g. broiler litter, laying poultry litter, cattle FYM, cattle slurry, pig FYM, pig slurry, sewage sludge…) | - |
|  | Volume of organic fertiliser applied at each fertiliser event | m3 ha-1 |
|  | Dry matter content of organic fertiliser at each fertiliser event | (% solids) or %MS |
|  | Estimated total N content of the organic fertiliser | (g N kg-1 dry matter) Kjeldahl or “Carbo-Erma” |
|  | Estimated total C content of the organic fertiliser | (g C kg-1 dry matter)** “Carbo-Erma” |
| Parameter Type | Parameter | Units |
| Fertiliser application (Organic) | Estimated available N (ammonium and nitrate) content of the organic fertiliser (most will be as ammonium) | (g available N kg-1 dry matter)* Extraction-colourimetric analysis |
|  | Estimated available C content of the organic fertiliser | (g available C kg-1 dry matter)* Extraction-Persulphate oxidation |
|  | Estimated N applied to surface from organic fertiliser for each application | (kg N ha-1) (calculate from volume applied, dry matter content and N content). |
|  | Estimated C applied to surface from organic fertiliser for each application | (kg C ha-1) |
|  | Application method of organic fertiliser | e.g. surface spreading, deep injection… |
| Other Treatments | Tillage | Dates of tillage |
|  | Depth of tillage | - |
|  | Pesticides | Note any dates of application, amount and type |
|  | Herbicides | Note any dates of application, amount and type |
|  | Irrigation | Note any dates of irrigation |
|  | Amount of water applied during irrigation | mm |
|  | Liming | Note any dates of liming, and amount of lime applied, and its elemental composition |

1c. Grassland Site Description & Management Record

| Parameter Type | Parameter | Units |
| --- | --- | --- |
| Management over longer period | Age of grassland (date last ploughed if not permanent) |  |
|  | Details of crops in the 2-3 years before field became grassland (if the grassland is < 5 years old) |  |
|  | Details of land use over the last 25 years. |  |
|  | Is the grassland grazed or used for hay/silage? |  |
|  | For cut grassland, number of cuts/year, yield per cut or per year on average |  |
|  | Is the plot drained and how ? |  |
|  | Fertilisation details over the last 5 years on average. #### Total kg N ha-1 yr-1: - mineral: - organic: |  |
|  | Any specific problems with the fields, water excess, soil conditions etc |  |
| Cutting | Dates of each cut | DD/MM/YY |
|  | Yield of each cut | kg m-2 |
|  | Height of vegetation before & after each cut * | M |
| Fertiliser application (mineral) | Dates of each mineral fertilisation | DD/MM/YY |
|  | Amount of N applied at each fertiliser event | (kg N ha-1) |
|  | Amount of phosphorus applied at each fertiliser event | (kg P ha-1) |
|  | Amount of potassium applied at each fertiliser event | (kg K ha-1) |
|  | Chemical form of mineral N, P and K applied (e.g. straight ammonium nitrate, urea, calcium ammonium nitrate, potassium phosphate…) | - |
| Parameter Type | Parameter Measured | Units |
| Fertiliser application (Organic) | Dates of each organic fertilisation | DD/MM/YY |
|  | Form of organic fertiliser applied at each fertiliser event (e.g. broiler litter, laying poultry litter, cattle FYM, cattle slurry, pig FYM, pig slurry, sewage sludge…) | - |
|  | Volume of organic fertiliser applied at each fertiliser event | m3 ha-1 |
|  | Dry matter content of organic fertiliser at each fertiliser event | (% solids) or %MS |
|  | Estimated total N content of the organic fertiliser | (g N kg-1 dry matter) Kjeldahl or “Carbo-Erma” |
|  | Estimated total C content of the organic fertiliser | (g C kg-1 dry matter)** “Carbo-Erma” |
|  | Estimated available N (ammonium and nitrate) content of the organic fertiliser (most will be as ammonium) | (g available N kg-1 dry matter)* Extraction-colourimetric analysis |
|  | Estimated available C content of the organic fertiliser | (g available C kg-1 dry matter)* Extraction-Persulphate oxidation |
|  | Estimated C applied to surface from organic fertiliser for each application | (kg C ha-1) |
|  | Application Method of organic fertiliser | e.g. surface spreading, deep injection… |
| Other Treatments | Tillage | Dates of tillage |
|  | Depth of tillage | - |
|  | Pesticides | Note any dates of application, amount and type |
|  | Herbicides | Note any dates of application, amount and type |
|  | Irrigation | Note any dates of irrigation |
|  | Amount of water applied during irrigation | - |
|  | Liming | Note any dates of liming, and amount of lime applied, and its elemental composition |
| Parameter Type | Parameter Measured | Units |
| Grazing | Continuous or rotational ? | - |
|  | Dates (keep log of grazing if it is changing during measurements) | DD/MM/YY |
|  | Density of animals (to note with dates on log) | (number/ha) |
|  | Estimated liveweight of animals, each month |  |
|  | Type of animals | - |

2. Fluxes, Concentrations and Meteorology

| Parameter Type | Parameter Measured | Symbol | Units | Method | Frequency | Duration (months) | Ecosystem Type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Above-canopy | Latent heat flux | LE | Wm-2 | Eddy cov | 30 min. | 15 | All |
|  | Carbon dioxide flux | Fc | µmol m-2 s-1 | Eddy cov | 30 min. | 15 | All |
|  | Sensible heat flux | H | Wm-2 | Eddy cov | 30 min. | 15 | All |
|  | Momentum flux |  | kg m-1 s-2 | Eddy cov | 30 min. | 15 | All |
|  | Nitric oxide concentration | NO | µmol m-3 | Chemiluminescent analysis | 30 min. | 15 | All |
|  | Nitrogen dioxide concentration | NO2 | µmol m-3 | Chemiluminescent analysis | 30 min. | 15 | All |
|  | Ozone concentration at height of O3 fast sensor | O3 | µmol m-3 | Chemiluminescent analysis | 30 min. | 15 | All |
|  | Ozone flux | F_O3 | µmol m-2 s-1 | Eddy cov (fast chemiluminescence) | 30 min | 15 | All |
|  | Ammonia flux | F_NH3 | µmol m-2 s-1 | Gradient (AMANDA/ GRAHAM/ROSAA) or Eddy cov (QCL, HT-CIRMS) | 30 min | IMPs | All |
|  | NO flux | F__EC_NO | nmol m-2 s-1 | Eddy cov | 30 min | IMPs | All |
|  | NO2 flux | F_EC_NO2 | nmol m-2 s-1 | Eddy cov | 30 min | IMPs | All |
|  | VOC flux | F_EC_VOC | nmol m-2 s-1 | (Disjunct) eddy cov (PTR-MS; FIS); REA | 30 min | IMPs | All |
|  | NO flux | F_grd_NO | µmol m-2 s-1 | Gradient | 30 min | Where available | All |
|  | NO2 flux | F_grd_NO2 | µmol m-2 s-1 | Gradient | 30 min | Where available | All |
|  | VOC flux | F_grd_NO3- | µmol m-2 s-1 | Gradient | 30 min | Where available | All |

| Wet Deposition | Bulk wet deposition | WD | mm | Bulk sampler | Monthly | 15 | All, nearby site |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bulk ammonium | WD_NH4+ | kg N ha-1 | Bulk sampler | Monthly | 15 | All, nearby site |
|  | Bulk nitrate | WD_NO3- | kg N ha-1 | Bulk sampler | Monthly | 15 | All, nearby site |
|  | Bulk sulphate | WD_SO4= | kg S ha-1 | Bulk sampler | Monthly | 15 | All, nearby site |
|  | Bulk chloride | WD_Cl- | kg Cl ha-1 | Bulk sampler | Monthly | 15 | All, nearby site |
|  | Leaf loading | LC_NH4+ | µmol m-2 | Leaf washing | Daily | IMPs, where NH3 flux measured | All sites, where NH3 flux measured |
|  | Leaf loading | LC_NO3- | µmol m-2 | Leaf washing | Daily |  |  |
|  | Leaf loading | LC_SO4= | µmol m-2 | Leaf washing | Daily |  |  |
|  | Leaf loading | LC_Cl- | µmol m-2 | Leaf washing | Daily |  |  |
|  | Leaf loading | LC_Na+ | µmol m-2 | Leaf washing | Daily |  |  |
|  | Leaf loading | LC_Mg++ | µmol m-2 | Leaf washing | Daily |  |  |
|  | Leaf loading | LC_K+ | µmol m-2 | Leaf washing | Daily |  |  |
|  | Leaf washing | LC_Ca++ | µmol m-2 | Leaf washing | Daily |  |  |
|  | Leaf washing | LC_pH | - | Leaf washing | Ideally daily |  |  |
| canopy measurements | In-canopy T profile | T_cp | °C | canopy air temperature at CO2 measurement heights | 30 min. | 15 | Forest |
|  | CO2 storage in canopy air layer | S_CO2 | mol m-2 s-1 | Profiles Li-COR 840 | 30 min. | 15 | Forest |
|  | In-canopy RH profile | RH_cp |  | Li-COR 840 or RH probe | 30 min |  |  |
|  | Leaf-level CO2 / H2O flux | gmax; Amax | mol m-2 s-1; µmol m-2 s-1 | Photosynthesis system Various leaf ages / positions | some intensive periods | IMPs | All, except very high species richness |
|  | O3 flux trunk space | Fc_O3 | µmol m-2 s-1 | Eddy-cov trunk space (fast chemoluminescence) | 30 min | IMPs | Forest / tall crops? |
| Soil Fluxes | Soil heat flux | G | W m-2 | Soil heat flux plates | 30 min. | 15 | All |
|  | Nitric Oxide | Fsoil_NO | nmol m-2 s-1 | Continuous auto chamber or EC + gradient | Min. 4/day | 15 | All |
|  | Nitrogen Dioxide | Fsoil_NO2 | nmol m-2 s-1 | Continuous auto chamber or EC + gradient | Min. 4/day | 15 | All |
|  | Ozone | Fsoil_O3 | µmol m-2 s-1 | Continuous auto chamber or gradient | Min. 4/day | 15 | All |
|  | Soil CO2 flux | Fsoil_CO2 | mol m-2 s-1 | Cuvette | Weekly with replicates | 15 | desirable where bare soil available |

| Meteorology | Global radiation | Rg | W m-2 | Sensor | 30 min | 15 | All |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Net radiation | Rn | W m-2 | Sensor | 30 min | 15 | All |
|  | 4 radiation components: (SW, LW in, out) | Rsw_in, Rsw_out, Rlw_in, Rlw_out | W m-2 | Sensor | 30 min | 15 | All |
|  | Photosynthetic photon flux density, total, direct, diffuse and reflected | PPFDtot PPFDdir PPFDdif PPFDref | mol m-2 s-1 | Sensor | 1 & 30 min | 15 | All |
|  | Ground-level PAR | PPFDg | mol m-2 s-1 | Sensor | 1 & 30 min | 15 | IT-Isp, NL-Hyy, F-Gri & others |
|  | Leaf wetness | LWet | % | Clip sensors (4 replicates) | 30 min | 15 | All |
|  | Air temperature | Ta | °C | Sensor | 30 min | 15 | All |
|  | Pressure | Pa | kPa | Sensor | 30 min | 15 | All |
|  | Bole temperature | TBOLE | °C | Sensor | 30 min | 15 | Forest where available |
|  | Precipitation | P | mm | Rain gauge | 30 min | 15 | All |
|  | Relative humidity | RH | % | Sensor | 30 min | 15 | All |
|  | Snow depth | SNOWD | mm | Sensor / Pole | 15 days (30 min), as relevant | 15 | All |

3. Soil & Vegetation

| Parameter Type | Parameter Measured | Symbol | Units | Method | Frequency | Duration (Months) | Ecosystem Type |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Soil | Soil description | SDESC | - | Survey | One-off |  | All |
|  | Depth of main rooting zone | DEPTHMRZ | m | Visual Observation | One-off |  | All |
|  | Bulk density | BD | g soil m-3 dry soil | Soil cylinders | One-off |  | All |
|  | Soil Texture (in each layer) | T | (Cl:Sa:Si) g/kg | Without decarbonatation | One-off |  | All |
|  | pH in water (in each layer) | pH | - |  | One-off |  | All |
|  | Water retention curve | Water content vs. water potential | % vs kPa |  | One-off |  | All |
|  | Soil temperature profile | Ts | °C | Sensor | 30 min, min. 2 to 5 depths | 15 | All |
|  | Soil water content profile & water table | SWC, SWD | % by volume | TDR | 30 min, min. 2 to 5 depths | 15 | All |
|  | Soil mineral N concentrations (NO3-, NH4+) |  | kg m-2 | Soil KCl extractions, 2 depths (0-5 cm; 0-30 cm) + litter where applicable | Site specific; at least monthly, more often after fertilisation | 15 | All |
|  | Soil organic C stocks (profile) | SOC | kg m-2 | Cores | one-off | once | All |
|  | Soil organic N stocks (profile) | SON | kg m-2 | Cores | one-off | once | All |
| Vegetation Vegetation | Mean canopy height | H | m | Direct / indirect sampling | Fortnightly, ecosystem dependent | 15 | All |
|  | Leaf area index (1/2 total) | LAI | m² m-2 | Direct / indirect sampling | Once per month or more frequently before and after cutting / grazing | 15 | All |
|  | Leaf area density profie | LAD | m2 m-3 | Spherical photography (forests); Canopy structure meter (others) | Every two months | 15 | All |
|  | Tissue C |  | % C dry matter | Carbo-Erma | Monthly | 15 | All |
|  | Tissue N |  | % N dry matter | Kjeldahl or Carlo Erba | Monthly + more frequently before and after management | 15 | All |
|  | Wood biomass | BW | kg m-2 | Inventory/sampling/site based allometric relationship | Once/twice/annually | 15 | Forest |
|  | Aboveground biomass | BAG | kg m-2 | Sampling | Annual/seasonal | 15 | Agricultural |
|  | Harvested biomass | BH | kg m-2 | Sampling / Statistic | If relevant/ at harvest | 15 | All |
|  | Residues after management | Res | kg m-2 |  | If relevant/at harvest | 15 | All |
|  | Yield |  |  |  | If relevant/at harvest | 15 | Agricultural |
|  | Plant species composition | - | % cover of each functional group | Sampling | Once / twice a year | 15 | All |
|  | Standing leaf biomass | BL | kg m-2 | Inventory/sampling/site based allometric relationship | Once / twice a year (Deciduous/Conifers) | 15 | Forest |
|  | Wood increment | WI | kg m-2 | Inventory, sampling, site-based allometry, dendrometers | Annual | 15 | Forest |
|  | Bulk foliar NH4+ | Bulk_NH4+ | µmol l-1 |  | Weekly & after management | IMPs | All with NH3 measurements |
|  | Bulk foliar NH4+ |  |  |  |  |  |  |
|  | Bulk foliar pH | Bulk_pH | - |  |  | IMPs |  |
|  | Litter NH4+ | Litter_NH4+ | µmol l-1 |  |  | IMPs |  |
|  | Litter pH | Litter_pH | - |  |  | IMPs |  |
|  | Apoplastic NH4+ | Apo_NH4+ | µmol l-1 | Vacuum infilitration |  | IMPs | Where possible |
|  | Apoplastic pH | Apo_pH | - | Vacuum infilitration |  |  | Where possible |